CPE301 – SPRING 2019

# Design Assignment DA2B

Student Name: Cody McDonald

Student #: 5000382538

Student Email: mcdonc4@unlv.nevada.edu

Primary Github address: https://github.com/elev8rProcrastinator/submission_da.git

Directory: https://github.com/elev8rProcrastinator/submission_da/tree/master/DA2B

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

ATMega328P mini

Multi-shield

- INITIAL CODE OF TASK DA.2. ASSEMBLY AND C-CODE:

ASSEMBLY:

; CPE301_DA2_assembly_task2.asm

;

; Created: 3/2/2019 3:31:09 PM

; Author : Cody

; TASK2

;TCNT for 1.25s = 0x4C4A

.ORG 0x00			;start memory at 0x00

LDI R20,5			;set clock pre-scaler to 1024 and normal mode

STS TCCR1B,R20		;attach value to correct register

CBI DDRC, 2			;set pin 2 to an input

LDI	R16,0b00111100	;set led pins to outputs

OUT DDRB, R16			;attach to ddr

LDI R16, 0xFF			;make sure all leds are off

OUT PORTB, R16

AGAIN:

LDI R16, 0b00111100		;turn led's off

OUT PORTB, R16			;output to port b

SBIC PINC,2				;skip next line if switch 2 is pressed

RJMP OVER				;jump to over subroutine

LDI R16, 0b00111000		;turn on led

OUT PORTB, R16			;output to port

LDI R20,0x00			;reset the counter to 0

STS TCNT1H,R20

STS TCNT1L,R20

RCALL DELAY				;go to delay subroutine

RJMP AGAIN

OVER:					;subroutine to loop if button isn't pressed

SBI PORTB,2

RJMP AGAIN

DELAY:

LDS R29, TCNT1H		;loading upper bit of counter to R29

LDS R28, TCNT1L		;loading lower bit of counter to R28

CPI R28,0x4A			;comparing if lower 8 bits of timer is 0x4A

BRSH BODY				;if lower bits of timer have reached desired amount, check the upperbits

RJMP DELAY				;otherwise, keep checking lower bits

BODY:

CPI R29,0x4C			;check to see if upper timer bits have reached the desired value

BRLT DELAY				;if not, recheck the lower bits

RET					;once the timer reached the desired value, toggle the LED

C-CODE:

/*

* CPE301_DA2_C_task2.c

*

* Created: 3/2/2019 12:04:23 AM

* Author : Cody

*/

#define F_CPU 16000000UL //set clock speed to 16MHz

//include important header files

#include <avr/io.h>

#include <stdio.h>

#include <util/delay.h>

int main(void){

//initialize registers

DDRB |= (1<<2);		//set portb 2 to output

PORTB |= (1<<2);		//set pb2 to high

DDRC &= (0<<2);		//set ddrc to input

PORTC |= (0<<2);		//set portc2 to low

while (1) {

if (!(PINC & (1 << PINC2))) //check for button press

{

PORTB &= ~(1<<2);		//if pressed then turn on led

_delay_ms(1250);		//keep led on for 1.25s

}

else {

PORTB |= (1<<2);		//if not pressed then keep led off

}

}

return 0;

}

- DEVELOPED MODIFIED CODE OF TASK DA.2. ASSEMBLY AND C-CODE:

Assembly:

- ;

- ; CPE301_DA2B_assembly_task2.asm

- ;

- ; Author : Cody

- ; TASK2

- .include <m328pdef.inc>

- .ORG 0x00			;start memory at 0x00

- JMP MAIN

- .ORG 0X10			;jump memory back to 0x10

- JMP PCINT1_ISR

- MAIN:

- ;Set stack pointers

- LDI R20, HIGH(RAMEND)

- OUT SPH, R20

- LDI R20, LOW(RAMEND)

- OUT	SPL, R20

- ;Initialize ports

- LDI R17, 0b0011_1000	;reset register

- LDI R19, 4				;used to toggle 4th LED

- LDI R21, 0xFF			;used to set DDRB high

- OUT DDRB, R21		;set led ports to high

- OUT	PORTB, R21		;set leds to low

- OUT	DDRC, R19		;set ddrc to inputs

- ;Set modes for interrupt

- LDI R20,0b0000_0100

- STS PCMSK1, R20

- OUT PORTC, R20

- LDI R20, (1<<PCIE1)

- STS PCICR, R20

- SEI					;start interrupt

- ;Run an infinite loop

- HERE:

- OUT PORTB,R21 ;keep LED off unless

- JMP HERE

- ;interrupt subroutine

- PCINT1_ISR:

- EOR R17,R19

- OUT PORTB,R17

- ;delay subroutine

- ldi  r22, 102

- ldi  r23, 118

- ldi  r24, 194

- L1: dec  r24

- brne L1

- dec  r23

- brne L1

- dec  r22

- brne L1

- RETI	;return back to loop

C-Code:

/*

* CPE301_DA2B_C.c

*

* Created: 3/2/2019 12:04:23 AM

* Author : Cody

*/

#define F_CPU 16000000UL //set clock speed to 16MHz

//include important header files

#include <avr/io.h>

#include <stdio.h>

#include <util/delay.h>

#include <avr/interrupt.h>

int main(void){

//initialize registers

DDRB |= (1<<2)|(1<<5);

PORTB |= (1<<5);		//turn off LED

PORTB |= (1<<2);		//set pb2 to high

DDRC &= (0<<1)|(0<<2)|(0<<3);		//set ddrc to input

PORTC |= (1<<1)|(1<<2)|(1<<3);		//set portc 1/2/3 to high

// Enable PC at pins 9/10/11

PCMSK1 |= (1<<PCINT9)|(1<<PCINT10)|(1<<PCINT11); //set corresponding pins in msk

PCICR |= (1<<PCIE1);							// turn them on in register

sei();						// start interrupt

while (1) ;

}

ISR (PCINT1_vect){			//delay subroutine

PORTB ^= (1<<2);

_delay_ms(1250);

}

- SCHEMATICS

N/A

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

TASK 1/DA2B Assembly:

Figure 1: The button is set high and remains outside of the interrupt loop

Figure 2: after button is depressed the code will enter the delay subroutine

Figure 3: This shows the amount of delay in the delay subroutine which adds up to 1.25s

TASK 1/DA2B C-Code:

Figure 4:Before the button is depressed the code remains outside of the delay and LED subroutine

Figure 5: Afterwards the the button is depressed and enters the subroutine. The delay function creates a 1.25ms delay which I believe doesn't need to be proven to show the delay.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

The board setup is the same as DA2, so I will use the same screen shot. The same button must be pressed along with the same LED lighting up.

Below is the multi-shield sitting on top of the AVR. The two pictures demonstrate the effect of a button press for the second task.

- VIDEO LINKS OF EACH DEMO

Both  demos are featured in the below link.

https://www.youtube.com/watch?v=uRLvSl80LxY

- GITHUB LINK OF THIS DA

https://github.com/elev8rProcrastinator/submission_da/tree/master/DA2B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Cody McDonald