CPE301 – SPRING 2019

# Design Assignment DA3A

Student Name: Cody McDonald

Student #: 5000382538

Student Email: mcdonc4@unlv.nevada.edu

Primary Github address: https://github.com/elev8rProcrastinator/submission_da.git

Directory: https://github.com/elev8rProcrastinator/submission_da/tree/master/DA3A

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

AtMini xplained

FTDI USB to TTl communication

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* CPE301_DA3A.c

*

* Created: 3/27/2019 7:10:08 PM

* Author : Cody McDonald

*/

#define F_CPU 16000000UL

#define BAUD 9600

#include <util/setbaud.h>

#include <avr/io.h>

#include <stdio.h>

#include <util/delay.h>

#include <avr/interrupt.h>

void USART_tx_data(char data);			// Function to send integer/char

void USART_tx_string(char* StringPtr);	// Function to send string

void USART_init(void);					// Function to initialize the UART communication

char str_type[] = "String: ";		// Declaring the string value on screen

char int_type[] = "Integer: ";		// Declaring the integer value on screen

char float_type[] = "Floating Point: "; // Declaring the floating point value on screen

char line[] = "\n";					// Used to create the next line

char string[] = "I love eggs";		//String[] is the variable to output into terminal

char output[20];					// Allocating memory space to contain the float value

volatile float adc_temp = 100.00;	// Sets the float value

int main(void)

{

TCCR1B |= 5;		// Sets prescaler to 1024

TIMSK1 = (1 << TOIE1); // Enables overflow flag

TCNT1 = 49911;		// Set top of timer for a delay of 1 second

USART_init();		// Initializes the analog to digital functions as well as OVF interrupt

sei();				//enable interrupts

while(1);			//infinite loop

}

ISR (TIMER1_OVF_vect)

{

USART_tx_string(line);			// go to next line

USART_tx_string(str_type);		// Label the string

USART_tx_string(string);		// prints string

USART_tx_string(line);			// creates next line

USART_tx_string(int_type);		// Label the integer

USART_tx_data('8');				// prints value a numerical value

USART_tx_string(line);			// creates next line

USART_tx_string(float_type);	// Label the FLoating Point

snprintf(output, sizeof(output), "%f\r\n", adc_temp); // output everything

USART_tx_string(output);		// transmits outs to UART

USART_tx_string(line);			// creates next line

TCNT1 = 49911;					// reset top of counter

}

//Function to initialize UART

void USART_init( void )

{

UBRR0H = 0;

UBRR0L = F_CPU/16/BAUD - 1; // Used for the BAUD prescaler

UCSR0C = _BV(UCSZ01) | _BV(UCSZ00); /* 8-bit data */

UCSR0B = _BV(RXEN0) | _BV(TXEN0);   /* Enable RX and TX */

}

//create function to send character data

void USART_tx_data(char data)

{

while (!(UCSR0A & (1 << UDRE0)));	// Set proper modes for UARRT

UDR0 = data;					// grabs the value given from data

}

//create function to send strings

void USART_tx_string(char *StrPtr)

{

while ((*StrPtr != '\0')){				// loop until end of line

while (!(UCSR0A & (1 << UDRE0)));	// loop until UDRE0 is high

UDR0 = *StrPtr;					// Store value intp UDR0

StrPtr++;					// increase to next position

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A – there was no modified code

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Figure 1:Screenshot of Atmel output

Figure 2: Screenshot of PuTTy output

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=wlVayFI1Ag0

- GITHUB LINK OF THIS DA

https://github.com/elev8rProcrastinator/submission_da/tree/master/DA3A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Cody McDonald