CPE301 – SPRING 2019

# Design Assignment DA3A

Student Name: Cody McDonald

Student #: 5000382538

Student Email: mcdonc4@unlv.nevada.edu

Primary Github address: https://github.com/elev8rProcrastinator/submission_da.git

Directory: https://github.com/elev8rProcrastinator/submission_da/tree/master/DA3B

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

AtMini xplained

FTDI USB to TTl communication

LM35 temperature sensor

Block diagram with pins used in the Atmega328P

Figure 1: LM35 pin list

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* The program reads the LM34 temperature of the MCU using ADC and sends it to the PC.

* If you put your finger on the MCU, the number increases.

*

*/

#define F_CPU 16000000UL

#define BAUD_RATE 9600

#include <avr/io.h>

#include <util/delay.h>

void usart_init ();

void usart_send (unsigned char ch);

int main (void)

{

usart_init ();

/** Setup and enable ADC **/

ADMUX = (0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(1<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC5 PC5

(1<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC ENable

(0<<ADSC)|     // ADC Start Conversion

(0<<ADATE)|    // ADC Auto Trigger Enable

(0<<ADIF)|     // ADC Interrupt Flag

(0<<ADIE)|     // ADC Interrupt Enable

(1<<ADPS2)|    // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

while (1)

{

ADCSRA|=(1<<ADSC);	//start conversion

while((ADCSRA&(1<<ADIF))==0);//wait for conversion to finish

ADCSRA |= (1<<ADIF);

int a = ADCL;

a = a | (ADCH<<8);

a = (a/1024.0) * 5000/10;

a = (a*2)+32; //equation to convert celsius to farenheit. can't use used 2 instead of 9/5

usart_send((a/100)+'0');

a = a % 100;

usart_send((a/10)+'0');

a = a % 10;

usart_send((a)+'0');

usart_send('\r');

_delay_ms(1000);

}

return 0;

}

void usart_init (void)

{

UCSR0B = (1<<TXEN0);

UCSR0C = (1<< UCSZ01)|(1<<UCSZ00);

UBRR0L = F_CPU/16/BAUD_RATE-1;

}

void usart_send (unsigned char ch)

{

while (! (UCSR0A & (1<<UDRE0))); 	//wait until UDR0 is empty

UDR0 = ch;							//transmit ch

}

void usart_print(char* str)

{

int i = 0;

while(str[i] != 0)

usart_send(str[i]);

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A – there was no modified code

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Figure 2:Temparture sensor readings in time

Figure 3: Temperatures read from the terminal

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Figure 4:Set up to read from LM35 sensor

- VIDEO LINKS OF EACH DEMO

https://youtu.be/u4hgoxbDR34

- GITHUB LINK OF THIS DA

https://github.com/elev8rProcrastinator/submission_da/tree/master/DA3B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Cody McDonald