CPE301 – SPRING 2019

# Design Assignment DA5

Student Name: Cody McDonald

Student #: 5000382538

Student Email: mcdonc4@unlv.nevada.edu

Primary Github address: https://github.com/elev8rProcrastinator/submission_da.git

Directory: https://github.com/elev8rProcrastinator/submission_da/tree/master/DA5

Partner: Christopher Barr

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmini Xplained

LM35

FTDI USB to serial converter

NRF241L01

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

|  | // MIT License |
| --- | --- |
|  | // |
|  | // Copyright (c) 2018 Helvijs Adams |
|  | // |
|  | // Permission is hereby granted, free of charge, to any person obtaining a copy |
|  | // of this software and associated documentation files (the "Software"), to deal |
|  | // in the Software without restriction, including without limitation the rights |
|  | // to use, copy, modify, merge, publish, distribute, sublicense, and/or sell |
|  | // copies of the Software, and to permit persons to whom the Software is |
|  | // furnished to do so, subject to the following conditions: |
|  | // |
|  | // The above copyright notice and this permission notice shall be included in all |
|  | // copies or substantial portions of the Software. |
|  | // |
|  | // THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR |
|  | // IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, |
|  | // FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE |
|  | // AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER |
|  | // LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, |
|  | // OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE |
|  | // SOFTWARE. |
|  | // |
|  | // Software was tested on ATmega328P and ATmega328PB (PB needs few changes in SPI) |
|  | // RF module software was tested on - cheap nRF24L01+ from China |
|  | // All the relevant settings are defined in nrf24l01.c file |
|  | // Some features will be added later, at this moment it is bare minimum to send/receive |
|  | // |
|  | // Set clock frequency |
|  | #ifndef F_CPU |
|  | #define F_CPU 16000000UL |
|  | #endif |
|  | #include <avr/io.h> |
|  | #include <util/delay.h> |
|  | #include <avr/interrupt.h> |
|  | #include <stdbool.h> |
|  | #include <stdio.h> |
|  | #include <string.h> |
|  | // Set up UART for printf(); |
|  | #ifndef BAUD |
|  | #define BAUD 9600 |
|  | #endif |
|  | #include "inc\STDIO_UART.h" |
|  | // Include nRF24L01+ library |
|  | #include "inc\nrf24l01.h" |
|  | #include "inc\nrf24l01-mnemonics.h" |
|  | #include "inc\spi.h" |
|  | void print_config(void); |
|  | // Used in IRQ ISR |
|  | volatile bool message_received = false; |
|  | volatile bool status = false; |
|  | int main(void) |
|  | { |
|  | // Set cliche message to send (message cannot exceed 32 characters) |
|  | char tx_message[32]; // Define string array |
|  | strcpy(tx_message,"Hello World!"); // Copy string into array |
|  | // Initialize UART |
|  | uart_init(); |
|  | // Initialize nRF24L01+ and print configuration info |
|  | nrf24_init(); |
|  | print_config(); |
|  | // Start listening to incoming messages |
|  | nrf24_start_listening(); |
|  | while (1) |
|  | { |
|  | if (message_received) |
|  | { |
|  | // Message received, print it |
|  | message_received = false; |
|  | printf("Received message: %s\n",nrf24_read_message()); |
|  | // Send message as response |
|  | _delay_ms(500); |
|  | status = nrf24_send_message(tx_message); |
|  | if (status == true) printf("Message sent successfully\n"); |
|  | } |
|  | } |
|  | } |
|  | // Interrupt on IRQ pin |
|  | ISR(INT0_vect) |
|  | { |
|  | message_received = true; |
|  | } |
|  | void print_config(void) |
|  | { |
|  | uint8_t data; |
|  | printf("Startup successful\n\n nRF24L01+ configured as:\n"); |
|  | printf("-------------------------------------------\n"); |
|  | nrf24_read(CONFIG,&data,1); |
|  | printf("CONFIG 0x%x\n",data); |
|  | nrf24_read(EN_AA,&data,1); |
|  | printf("EN_AA 0x%x\n",data); |
|  | nrf24_read(EN_RXADDR,&data,1); |
|  | printf("EN_RXADDR 0x%x\n",data); |
|  | nrf24_read(SETUP_RETR,&data,1); |
|  | printf("SETUP_RETR 0x%x\n",data); |
|  | nrf24_read(RF_CH,&data,1); |
|  | printf("RF_CH 0x%x\n",data); |
|  | nrf24_read(RF_SETUP,&data,1); |
|  | printf("RF_SETUP 0x%x\n",data); |
|  | nrf24_read(STATUS,&data,1); |
|  | printf("STATUS 0x%x\n",data); |
|  | nrf24_read(FEATURE,&data,1); |
|  | printf("FEATURE 0x%x\n",data); |
|  | printf("-------------------------------------------\n\n"); |
|  | } |

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

// 	MIT License

//

// 	Copyright (c) 2018 Helvijs Adams

//

// 	Permission is hereby granted, free of charge, to any person obtaining a copy

// 	of this software and associated documentation files (the "Software"), to deal

// 	in the Software without restriction, including without limitation the rights

// 	to use, copy, modify, merge, publish, distribute, sublicense, and/or sell

// 	copies of the Software, and to permit persons to whom the Software is

// 	furnished to do so, subject to the following conditions:

//

// 	The above copyright notice and this permission notice shall be included in all

// 	copies or substantial portions of the Software.

//

// 	THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR

// 	IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY,

// 	FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE

// 	AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER

// 	LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM,

// 	OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE

// 	SOFTWARE.

//

//	Software was tested on ATmega328P and ATmega328PB (PB needs few changes in SPI)

//	RF module software was tested on - cheap nRF24L01+ from China

//	All the relevant settings are defined in nrf24l01.c file

//	Some features will be added later, at this moment it is bare minimum to send/receive

//

//	Set clock frequency

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdio.h>

#include <string.h>

//	Set up UART for printf();

#ifndef BAUD

#define BAUD 9600

#endif

#include "lib\STDIO_UART.c"

//	Include nRF24L01+ library

#include "lib\nrf24l01.c"

#include "lib\nrf24l01-mnemonics.h"

#include "lib\spi.c"

void print_config(void);

//	Used in IRQ ISR

volatile bool message_received = false;

volatile bool status = false;

// Global variables

volatile uint8_t ADCdata;

volatile unsigned char temp[10];

int main(void)

{

// Set up ADC

ADMUX |= (1 << REFS0); // use AVcc

ADMUX |= (1 << ADLAR); // Right adjust

ADCSRA = (1 << ADEN)	// Enable

|(1 << ADPS1)

|(1 << ADPS0)	// 128 prescaler for 16Mhz

|(1 << ADATE)	// ADC Auto Trigger

|(1 << ADSC);	// Start ADC

//	Set cliche message to send (message cannot exceed 32 characters)

char tx_message[32];				// Define string array

unsigned char i;

char dummy[10];

//	Initialize UART

uart_init();

//	Initialize nRF24L01+ and print configuration info

nrf24_init();

print_config();

//	Start listening to incoming messages

nrf24_start_listening();

strcpy(tx_message,"Chris, my bagels are wet");	// Copy string into array

nrf24_send_message(tx_message);

while (1)

{

ADCdata = (ADCH << 1) * 1.8 + 32; // Convert Celsius to Fahrenheit

itoa(ADCdata, dummy, 10); //convert char to ascii

for(i = 0 ; i < 10 ; i++){

temp[i] = dummy[i]; //move converted ascii

}

if (message_received)

{

//	Message received, print it

message_received = false;

printf("Received message: %s\n",nrf24_read_message());

//	Send message as response

_delay_ms(500);

status = nrf24_send_message(temp);

if (status == true) printf("Message sent successfully\n");

}

}

}

//	Interrupt on IRQ pin

ISR(INT0_vect)

{

message_received = true;

}

void print_config(void)

{

uint8_t data;

printf("Startup successful\n\n nRF24L01+ configured as:\n");

printf("-------------------------------------------\n");

nrf24_read(CONFIG,&data,1);

printf("CONFIG		0x%x\n",data);

nrf24_read(EN_AA,&data,1);

printf("EN_AA			0x%x\n",data);

nrf24_read(EN_RXADDR,&data,1);

printf("EN_RXADDR		0x%x\n",data);

nrf24_read(SETUP_RETR,&data,1);

printf("SETUP_RETR		0x%x\n",data);

nrf24_read(RF_CH,&data,1);

printf("RF_CH			0x%x\n",data);

nrf24_read(RF_SETUP,&data,1);

printf("RF_SETUP		0x%x\n",data);

nrf24_read(STATUS,&data,1);

printf("STATUS		0x%x\n",data);

nrf24_read(FEATURE,&data,1);

printf("FEATURE		0x%x\n",data);

printf("-------------------------------------------\n\n");

}

- SCHEMATICS

Refer to board demos for connections of the motors and drivers to the pins on the microcontroller

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Terminal window of the communication between the two microcontrollers.

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

This is the set-up of my controller setup and the microcontroller I transferred data to

- VIDEO LINKS OF EACH DEMO

https://youtu.be/qpUf0vQSzPM

- GITHUB LINK OF THIS DA

https://github.com/elev8rProcrastinator/submission_da/tree/master/DA5

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Cody McDonald