CPE301 – SPRING 2019

# Design Assignment DA6

Student Name: Cody McDonald

Student #: 5000382538

Student Email: mcdonc4@unlv.nevada.edu

Primary Github address: https://github.com/elev8rProcrastinator/submission_da.git

Directory: https://github.com/elev8rProcrastinator/submission_da/tree/master/DA6

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmini Xplained

LM35

FTDI USB to serial converter

MPU-6050:

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

//include standard libraries

#include <avr/io.h>

#include <stdlib.h>

#include <stdio.h>

#include <util/delay.h>

#include <math.h>

//include custom libraries

#include "MPU6050_def.h"

#include "i2c_master.h"

#include "uart.h"

//set bits for reading and writing with the mpu6050

#define MPU6050_WRITE 0xD0

#define MPU6050_READ 0xD1

//Global variables

float Acc_x, Acc_y, Acc_z;

float Gyro_x, Gyro_y, Gyro_z;

float temp;

//Function declarations

void getValues(void);

void uart_char(unsigned char c);

void uart_string(char *s);

int main(void){

char buffer[20];

char floatVal[10];

float Ax, Ay, Az;

float Gx, Gy, Gz;

//initialize our communication modes and gyroscope

i2c_init();

init_MPU6050();

init_uart(9600);

while(1){

getValues(); //get raw values

//covert each raw value with their correct divisor

Ax = Acc_x/16384.0;

Ay = Acc_y/16384.0;

Az = Acc_z/16384.0;

Gx = Gyro_x/16.4;

Gy = Gyro_y/16.4;

Gz = Gyro_z/16.4;

USART_SendString("\n-----------------------\n");//Spacer line

//output Ax value

dtostrf( Ax, 3, 2, floatVal );

sprintf(buffer,"Ax = %s g, ",floatVal);

USART_SendString(buffer);

//output Ay value

dtostrf( Ay, 3, 2, floatVal );

sprintf(buffer,"Ay = %s g, ",floatVal);

USART_SendString(buffer);

//output Az value

dtostrf( Az, 3, 2, floatVal );

sprintf(buffer,"Az = %s g\n\n",floatVal);

USART_SendString(buffer);

//output Gx value

dtostrf( Gx, 3, 2, floatVal );

sprintf(buffer,"Gx = %s degrees/s, ",floatVal);

USART_SendString(buffer);

//output Gy value

dtostrf( Gy, 3, 2, floatVal );

sprintf(buffer,"Gy = %s degrees/s, ",floatVal);

USART_SendString(buffer);

//output Gz value

dtostrf( Gz, 3, 2, floatVal );

sprintf(buffer,"Gz = %s degrees/s",floatVal);

USART_SendString(buffer);

USART_SendString("\n-----------------------\n"); //end line break

_delay_ms(1000);

}

return 0;

}

void init_uart(uint16_t baudrate){

uint16_t UBRR_val = (F_CPU/16)/(baudrate-1);

UBRR0H = UBRR_val >> 8;

UBRR0L = UBRR_val;

UCSR0B |= (1<<TXEN0) | (1<<RXEN0) | (1<<RXCIE0);

UCSR0C |= (1<<USBS0) | (3<<UCSZ00);

}

void uart_char(unsigned char c){

while(!(UCSR0A & (1<<UDRE0))); // wait

UDR0 = c; // output character

}

void uart_string(char *s){

while(*s){

uart_char(*s);

s++;

}

}

void init_MPU6050(void){

_delay_ms(150);

i2c_start(MPU6050_WRITE); /* Start with device write address */

i2c_write(SMPLRT_DIV);  /* Write to sample rate register */

i2c_write(0x07);		/* 1KHz sample rate */

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(PWR_MGMT_1); /* Write to power management register */

i2c_write(0x01); /* X axis gyroscope reference frequency */

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(CONFIG); /* Write to Configuration register */

i2c_write(0x00);	/* Fs = 8KHz */

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(GYRO_CONFIG); /* Write to Gyro configuration register */

i2c_write(0x18);		/* Full scale range +/- 2000 degree/C */

i2c_stop();

i2c_start(MPU6050_WRITE);

i2c_write(INT_ENABLE);  /* Write to interrupt enable register */

i2c_write(0x01);

i2c_stop();

}

void getValues(void){

//Start system by setting cursor

i2c_start(MPU6050_WRITE);

i2c_write(ACCEL_XOUT_H);

i2c_stop();

//Start reading process for each value

i2c_start(MPU6050_READ);

Acc_x = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

Acc_y = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

Acc_z = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

temp = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

Gyro_x = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

Gyro_y = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

Gyro_z = (((int)i2c_read_ack()<<8) | (int)i2c_read_ack());

//Stop reading

i2c_stop();

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

Refer to board demos for connections of the motors and drivers to the pins on the microcontroller

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Terminal window of my UART communication

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

This is the set-up of my controller setup

- VIDEO LINKS OF EACH DEMO

https://youtu.be/YTTyRZ8qZz4

- GITHUB LINK OF THIS DA

https://github.com/elev8rProcrastinator/submission_da/tree/master/DA6

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Cody McDonald