CPE301 – SPRING 2019

# MIDTERM 1

Student Name: Cody McDonald

Student #:5000382538

Student Email: mcdonc4@unlv.nevada.edu

Primary Github address: https://github.com/elev8rProcrastinator

Directory: https://github.com/elev8rProcrastinator/submission_da/tree/master/DA_Midterm

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/Midterm, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

AtMini xplained:

ESP8266:

LM35 temperature sensor:

Connected setup will be located in schematic description below.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* CPE301_Midterm_V3.c

*

* Created: 4/6/2019 4:12:34 PM

* Author : Cody

*/

#define F_CPU 16000000UL

//Include Header Files

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <stdint.h>

#include <stdlib.h>

// Global constants for uart

#define BAUD 115200

#define FOSC 16000000

#define UBRR FOSC/8/BAUD-1

// Global variables

volatile uint8_t ADCdata;

volatile unsigned char temp[10];

//AT COMMAND STRINGS

volatile unsigned char AT[] = "AT\r\n"; // Test

volatile unsigned char CWMODE[] = "AT+CWMODE=3\r\n"; // Set Wi-Fi mode

volatile unsigned char CWJAP[] = "AT+CWJAP=\"Monarchs Palace\",\"quackquack\"\r\n"; // Get Wi-Fi info

volatile unsigned char CIPSTART[] = "AT+CIPSTART=\"TCP\",\"184.106.153.149\",80\r\n"; // Establish connection with ThingSpeak

volatile unsigned char CIPSEND[] = "AT+CIPSEND=50\r\n"; // Send Temperature

volatile unsigned char CIPMUX[] = "AT+CIPMUX=0\r\n"; // Enable connection

volatile unsigned char SEND_DATA[] = "GET /update?key=ZDG1BP942G9NVEWD&field1="; // Get Write Key

volatile unsigned char RESET[] = "AT+RST\r\n"; // Get AT Firmware info

volatile unsigned char LINEBREAK[] = "\r\n"; // end of temperature transmission

volatile unsigned char CLOSE[] = "AT+CIPCLOSE\r\n";

//initialize functions

void ADC_init();

void usart_init();

void usart_send(volatile unsigned char data[]);

int main(void)

{

// initialize modes

ADC_init();		//initialize ADC

usart_init();	//initialize usart

sei();

// Establish initial connection the thingspeak

_delay_ms(1000);

usart_send(RESET);		//reset ESP

_delay_ms(1000);

usart_send(AT);			//confirm communication

_delay_ms(1000);

usart_send(CWMODE);		//WiFi mode = 3

_delay_ms(1000);

usart_send(CWJAP);		//Send wifi login

_delay_ms(1000);

usart_send(CIPMUX);		//Single connection point

while (1)

{

_delay_ms(1000);

usart_send(CIPSTART);	// Connect to ThingSpeak

_delay_ms(1000);

usart_send(CIPSEND);	// Declare send length 50

_delay_ms(1000);

usart_send(SEND_DATA);	// Connect to proper key

usart_send(temp);		// Send adc data

usart_send(CLOSE);		// close connection

usart_send(LINEBREAK);	// line break

}

return 0;

}

void ADC_init(){

ADMUX |= (1 << REFS0); // use AVcc

ADMUX |= (1 << ADLAR); // Right adjust

ADCSRA = (1 << ADEN)	// Enable

|(1 << ADPS1)

|(1 << ADPS0)	// 128 prescaler for 16Mhz

|(1 << ADATE)	// ADC Auto Trigger

|(1 << ADIE)	// Enable Interrupts

|(1 << ADSC);	// Start ADC

}

void usart_init() {

UBRR0H = ((UBRR) >> 8);

UBRR0L = UBRR;

UCSR0A |= (1<< U2X0); // divisor baud = 8

UCSR0B |= (1 << TXEN0); // Enable transmition

UCSR0C |= (1 << UCSZ01) | (1 << UCSZ00); // 8 bits

}

// ADC value Interrupt subroutine

ISR(ADC_vect) {

unsigned char i;

char dummy[10];

ADCdata = (ADCH << 1) * 1.8 + 32; // Convert Celsius to Fahrenheit

itoa(ADCdata, dummy, 10); //convert char to ascii

for(i = 0 ; i < 10 ; i++){

temp[i] = dummy[i]; //move converted ascii

}

}

void usart_send(volatile unsigned char data[]) {

volatile unsigned char length = 0;

volatile unsigned char j;

// write char array to UART register

while(data[length] != 0){

length++;

for(j = 0 ; j < length ; j++)

{

while(!(UCSR0A & (1<<UDRE0)));	// UART

UDR0 = data[j];					// send char

}

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A there was no modified code.

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (THINGSPEAK OUTPUT)

Temperature data sent to Thingspeak:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/Z6EL1btv30s

- GITHUB LINK OF THIS DA

https://github.com/elev8rProcrastinator/submission_da/tree/master/DA_Midterm

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT